# Corps Shoaling Analysis Tool (CSAT) Setup Instructions

Updated 27 Feb 2025

# Introduction

The Corps Shoaling Analysis Tool (CSAT) is a tool designed to support engineers, scientists, and decision makers explore and better understand shoaling rates calculated from collections of hydrographic survey data. The shoaling rates identified by CSAT can be saved and reported to the user in a variety of formats (e.g., tabular spreadsheets and raster products) that may support further analysis efforts. In addition to computing shoaling rates, CSAT also generates dredging volume estimates based on existing conditions.

Earlier versions of CSAT were distributed as compiled MATLAB executables. The MATLAB executable versions of CSAT (i.e. all versions < 2.0) have been deprecated and are no longer supported. Instead, with the release of CSAT v2.0, CSAT has been ported to the Python programming language. The Anaconda Distribution for Python, which is freely available for download, is recommended for preparing the CSAT workspace. CSAT has been tested to work with Python 3.7 and 3.8 on a Windows platform. It might be possible for CSAT to work with other versions of Python on different platforms, but additional steps for configuration might be necessary. Although CSAT has been created using the Python programming language, there is no requirement for the user to have any experience using Python.

This document covers the functional aspects of downloading the Anaconda Distribution for Python, obtaining CSAT, and preparing the workspace for performing shoaling analysis with CSAT.

# Step 1: Check for existing installations of Anaconda

CSAT was developed to work in an environment with a particular set of Python packages. The recommended way to create this environment is to use conda, a tool that is installed as part of Anaconda, a popular distribution of Python for data science.

Prior to downloading Anaconda, check to see if it is already installed on your computer.

- From the Start menu, look for an Anaconda3 folder and open an Anaconda prompt. Enter the command “conda --version” (without quotes)

  - If Anaconda is already installed on your system, you will see something like the screenshot below and skip to Step 3 to download CSAT:

  - If Anaconda is not installed, you will see a message like…

‘conda’ is not recognized as an internal or external command, operable program or batch file.

You will need to proceed to Step 2 to download and install Anaconda.

# Step 2: (If needed) Download and Install Anaconda

Anaconda is available to download free from the Anaconda website (https://www.anaconda.com/products/individual). Download the installer that is appropriate for your computer’s operating system (32-Bit or 64-Bit).

If you are a Corps employee working in the Corpnet computer, Anaconda is no longer available for installation due to the requirement for payment for a license.  As an alternative, we can use the version of Anaconda which comes installed with ESRI ArcGIS Pro.

- First, search for and request the ‘Microsoft .NET Desktop Runtime 8.0.5’ pre-requisite from the App Portal. This needs to be fully installed prior to requesting the next program.

- Second, search for and request the “ESRI ArcGIS Pro 3.4” version from the App Portal.

Once the installation process has completed, you should be able to open a command prompt, enter the command “conda –version” (without the quotation marks), and receive a similar response to the screenshot above.

# Step 3: Downloading CSAT

The current version of CSAT (Version 2.6.4 as of September 12, 2024) is available for download from the CSAT information page on the Coastal Inlets Research Program website (https://cirp.usace.army.mil/products/csat.php).

- In the Installation Instructions section, click the Green button in step three to download a zip file containing CSAT.

- Save the zip file to a location of your choosing.

- Once the zip file has finished downloading, extract the contents of the zip file.

- After the files have finished extracting, you should have a folder called CSAT_distribution. Within CSAT_distribution, there should be three subfolders (data, output, source_code) and three files (csat_env37.yml, Run_CSAT.bat, and Setup_CSAT.bat).

# Step 4: Setup the CSAT environment

In the directory where the zip folder was extracted, there is a batch file called Setup_CSAT.bat that will install the Python packages required by CSAT without interfering with any other Python packages already installed for other applications. Setup_CSAT.bat only needs run once. The time required for creating the CSAT environment can take anywhere from 3-15 minutes.

Note: When trying to run Setup_CSAT.bat, users on CorpsNet users may encounter a dialogue window that says that the batch script was unable to run because it was downloaded from the internet. One possible solution is right clicking the batch script, select Properties, and at the bottom of the Properties window is a checkbox option to consider the batch script as coming from a trusted source. Some users have reported that checking this box has permitted them to run the batch script successfully.

# Step 5: Obtaining CSAT Input Data

For CSAT to generate shoaling rate estimates, it needs repeat surveys of the study area. Fortunately, eHydro, an enterprise-level collection of hydrographic surveys, is an excellent and publicly available resource of USACE District surveys. CSAT expects the survey data and support files describing those surveys to be in a specific format.

The CSAT development team has already preprocessed nearly the entire collection of hydrographic surveys uploaded to eHydro. The preprocessed survey data is available for download from the CIRP website (https://cirp.usace.army.mil/products/csat_districts.php). Currently, the input survey data is updated on a quarterly basis to incorporate the latest surveys that have been uploaded to eHydro.

The CSAT input files can be downloaded for an entire USACE District at a time by clicking the Input link next to the desired District (see screenshot below). You will be prompted to save a zip file to your computer, and you should save the zip file to the data folder within CSAT_distribution where you previously extracted CSAT. Once the file download has completed, extract the contents of the zip file to the data folder.

Figure 2: CSAT input files for entire Districts are available for download from the CIRP website.

# Step 6: Running CSAT

After completing Steps 1-5, the only remaining task is to modify the runtime parameters in Run_CSAT.bat. In order to modify the batch script, you will need to open the file in a suitable text editor (e.g. Notepad or Notepad++). There are a few methods that you can open a batch script for editing. The first, and perhaps simplest way is to right click the batch script and select Edit from the context menu. If security settings prevent the batch script from opening, you can try the second method of changing the file extension from “.bat” to “.txt”. To do this, right click the file and select rename. Delete the bat extension and replace it with txt. When asked to confirm that you want to change the file extension, select Yes. Now you should be able to open the file in a text editor. Note: If you had to change the file extension to modify the batch script, once you are finished modifying the file, you will need to change the file extension back to .bat.

Once Run_CSAT.bat is open in the text editor, modify Line 10 to reflect the five-character code representing the District you wish to process. IMPORTANT: After changing Line 10, make sure that there are no spaces directly adjacent to the equals sign. If spaces are accidentally included, the batch script will fail to run and this can be a hard to diagnose reason.

Although not necessary, additional modifications can be made to Run_CSAT.bat to limit the shoaling analysis to specific periods of time, to enable/disable raster file output generation, or to modify additional input parameters to CSAT. An in-depth review of these additional options is beyond the scope of this current installation and setup guide. For more information, please refer to the CSAT User Guide (currently in development) or feel free to reach out to the CSAT development team.

After making changes to Run_CSAT.bat, save the changes and close the text editor.

Finally, double click the batch script to run CSAT. A command prompt window will open and text will begin to appear. The text reported in this window can be useful for providing ongoing run information to the user, but it is primarily used for debugging purposes. All of the text generated in the command prompt is automatically saved to a file called logging_output.log that can be found in the CSAT_distribution folder.

The tabular output data and raster files generated by CSAT are saved in the output subfolder.

The overall runtime for CSAT depends on several different factors. A large numbers of reaches being processed within the district, reaches with a large number of surveys available, and reaches with large spatial extents can all contribute to longer runtimes. Generally, computing the shoaling results for a single reach can take anywhere from 10 seconds to 2 minutes.