# WAM CY4.5 Output Documentation

WRITE(IU67,444)IT,IS,DATE(IT),NAME(IS),OUTLAT(IS),OUTLONG(IS),

&                SERIE(IT,1:3,IS), SERIE(IT,4,IS)*1000, SERIE(IT,14,IS),  &

&                SERIE(IT,7:13,IS), SERIE(IT,15:21,IS), SERIE(IT,23:29,IS)

444 FORMAT(i3,I3,1X,A14,1X,A6,1X,2F8.3,F6.1,F5.0,F5.2,F5.2,F5.2,3(5F6.2,2F5.0))

IT			Counter on output time

IS			Second counter on station

DATE:  			Year , month, day, hour, minute, second

NAME:			Station ID

XOUTLAT:		Latitude in degrees

XOUTLONG:		Longitude in degrees East

U10GR			Wind Speed in (m/s)

UDIR_GR		Wind Direction in (deg 0 from the north, clockwise)

US_GR			Frictional Velocity (m/s)

CD_GR:		Drag Coefficient * 1000 (unit-less)

TAUW_GR:		Wave Stress (m2/s2)

HS_GR:		Significant Wave Height Total  in (m)

PPER_GR:		Peak Spectral Wave Period Discrete Spectral Band (s)

MPER_GR:		Mean Wave Period (s)

Integral of 1.0/f-1E(f,θ)

TM1_GR:		First Moment Wave Period (s)

[Integral of fE(f,)  then inverted for period]

TM2_GR:	Second Moment Wave Period (s)

[ Integral of f2E(f,)  then inverted for period

MDIR_GR		Vector Mean Wave Direction in (deg 0 from the north, clockwise)

SPRE_GR		Directional Spread at the Mean Wave direction in (deg)

HS_SEA_GR		Same as above but for the wind sea portion

PPER_SEA_GR

MPER_SEA_GR

TM1_SEA_GR

TM2_SEA_GR

MDIR_SEA_GR

SPRE_SEA_GR

HS_SWELL_GR	Same as above but for the swell portion

PPER_SWELL_GR

MPER_SWELL_GR

TM1_SWELL_GR

TM2_SWELL_GR

MDIR_SWELL_GR

SPRE_SWELL_GR

ALL DIRECTIONS ARE IN METEOROLOGICAL COORDINATE SYSTEM:

0-deg from the north

90-deg from the east